Project Features:	Digital Input and Digital Output.

Program an Arduino board to accept a 4-bit binary input and display the numerical value (0 to 9) on a 7-Segment LED display.

Use the pushbutton for the digital input:

Use the 7-Segment LED Display:

Pin 1

Pin 5

Pin 10

Pin 6

To document your program, create a document and include the following sections in the document.  Provide a brief description of the system and how you are designing it to operate,

A.  Hardware Diagram:

Provide a hardware diagram of the components.

Analog Input

13

12

~ 11

~ 10

~ 9

8

7

~ 6

~ 5

4

~ 3

2

TX 1

RX 0

3.3V

5V

Gnd

Gnd

Vin

A0

A1

A2

A3

A4

A5

Power

Digital  Input / Output

Arduino UNO

Hardware Diagram

B.  Program Flowchart:

Draw a flowchart of the program using the Word graphics shapes.

Read Binary Input

Setup Input and

Output Pins

Y

Y

Y

Y

Y

Y

Y

Y

Y

Display “9”

Display “8”

Display “7”

Display “6”

Display “5”

Display “4”

Display “3”

Display “2”

Display “1”

Y

Display “0”

N

N

N

N

N

N

N

N

If 1001

If 1000

If 0111

If 0110

If 0101

If 0100

If 0011

If 0010

N

N

If 0001

If 0000

Display Blank

C.  Arduino Source Code

// Hardware Project 2 - 7-Segment Display with 4-bit Binary Input

void setup()

{

// Setup 7-segment display output pins

pinMode(13, OUTPUT); // segment g

pinMode(12, OUTPUT); // segment f

pinMode(11, OUTPUT); // segment e

pinMode(10, OUTPUT); // segment d

pinMode(9, OUTPUT);  // segment c

pinMode(8, OUTPUT);  // segment b

pinMode(7, OUTPUT);  // segment a

// Setup binary input pins

pinMode(5, INPUT); // bit 3

pinMode(4, INPUT); // bit 2

pinMode(3, INPUT); // bit 1

pinMode(2, INPUT); // bit 0

}

void loop()

{

// Read the 4-bit binary input and convert to decimal

int number = (digitalRead(5) << 3) | (digitalRead(4) << 2) | (digitalRead(3) << 1) | digitalRead(2);

// Display the corresponding number on the 7-segment display

if (number == 0)

{ digitalWrite(13, LOW); digitalWrite(12, HIGH); digitalWrite(11, HIGH); digitalWrite(10, HIGH);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, HIGH); }

else if (number == 1)

{ digitalWrite(13, LOW); digitalWrite(12, LOW); digitalWrite(11, LOW); digitalWrite(10, LOW);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, LOW); }

else if (number == 2)

{ digitalWrite(13, HIGH); digitalWrite(12, LOW); digitalWrite(11, HIGH); digitalWrite(10, HIGH);

digitalWrite(9, LOW); digitalWrite(8, HIGH); digitalWrite(7, HIGH); }

else if (number == 3)

{ digitalWrite(13, HIGH); digitalWrite(12, LOW); digitalWrite(11, LOW); digitalWrite(10, HIGH);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, HIGH); }

else if (number == 4)

{ digitalWrite(13, HIGH); digitalWrite(12, HIGH); digitalWrite(11, LOW); digitalWrite(10, LOW);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, LOW); }

else if (number == 5)

{ digitalWrite(13, HIGH); digitalWrite(12, HIGH); digitalWrite(11, LOW); digitalWrite(10, HIGH);

digitalWrite(9, HIGH); digitalWrite(8, LOW); digitalWrite(7, HIGH); }

else if (number == 6)

{ digitalWrite(13, HIGH); digitalWrite(12, HIGH); digitalWrite(11, HIGH); digitalWrite(10, HIGH);

digitalWrite(9, HIGH); digitalWrite(8, LOW); digitalWrite(7, HIGH); }

else if (number == 7)

{ digitalWrite(13, LOW); digitalWrite(12, LOW); digitalWrite(11, LOW); digitalWrite(10, LOW);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, HIGH); }

else if (number == 8)

{ digitalWrite(13, HIGH); digitalWrite(12, HIGH); digitalWrite(11, HIGH); digitalWrite(10, HIGH);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, HIGH); }

else if (number == 9)

{ digitalWrite(13, HIGH); digitalWrite(12, HIGH); digitalWrite(11, LOW); digitalWrite(10, HIGH);

digitalWrite(9, HIGH); digitalWrite(8, HIGH); digitalWrite(7, HIGH); }

else

{ digitalWrite(13, LOW); digitalWrite(12, LOW); digitalWrite(11, LOW); digitalWrite(10, LOW);

digitalWrite(9, LOW); digitalWrite(8, LOW); digitalWrite(7, LOW); }

}

D.  Demonstration Video

Record and upload a video demonstrating the operation of the circuit. (DONE)

Save the document as a PDF file and submit the PDF document to Blackboard.