Project Features:	Analog Input and Digital Output.

Program an Arduino board to accept an analog input voltage and display the magnitude of the voltage using a 10 LED display.

Input voltage = 0% of maximum (all LEDS off)

Input voltage = 30% of maximum

Input voltage = 70% of maximum

Document your program and include the following sections.  Provide a brief description of the system and how you are designing it to operate.

A.  Hardware Diagram:

Provide a hardware diagram of the components.

B.  Program Flowchart:

Draw a flowchart of the program.

C.  Arduino Source Code

Insert the Arduino Source Code into the document.

// Digital VU Meter - ECE 2305 Hardware Project 03

const int td = 100;

int analogVoltage = 0;

void setup() {

for (int pin = 4; pin <= 13; pin++) {

pinMode(pin, OUTPUT);

}

Serial.begin(9600);

}

void loop() {

analogVoltage = analogRead(A0);

Serial.println(analogVoltage);

int ledCount = map(analogVoltage, 0, 1023, 0, 10); // Map analog input to 0-10 LEDs

for (int i = 0; i < 10; i++) {

if (i < ledCount) {

digitalWrite(4 + i, HIGH); // Turn on LEDs up to ledCount

} else {

digitalWrite(4 + i, LOW);  // Turn off the rest

}

}

delay(td);

}

D.  Demonstration Video

Record and upload a video demonstrating the operation of the circuit.

DONE

Save the document as a PDF file and submit the PDF document to Blackboard.