Project Features:	Digital Output.

Program an Arduino board to simulate the traffic lights at the intersection of two streets.

R

R

Y

Y

G

G

North/South

East/West

Figure 1:  Traffic Lights

To document your program, create a Word document and include all of the following sections in the document.  Provide a brief description of the system and how you are designing it to operate,

A.  Hardware Diagram:

Provide a hardware diagram of the components.

Hardware Diagram

B.  Program Flowchart:

Draw a flowchart of the program using the Word graphics shapes.  Describe the timing of the traffic lights in the flowchart.

C.  Arduino Source Code

// Traffic Light Simulation

const int RED_NS = 10;   // North/South Red

const int YELLOW_NS = 9; // North/South Yellow

const int GREEN_NS = 8;  // North/South Green

const int RED_EW = 7;    // East/West Red

const int YELLOW_EW = 6; // East/West Yellow

const int GREEN_EW = 5;  // East/West Green

const unsigned long GREEN_TIME = 3000;  // ms

const unsigned long YELLOW_TIME = 1500; // ms

const unsigned long RED_TIME = 1200;    // ms

void initializeLights() {

// Set all traffic lights as OUTPUT

for (int pin = 5; pin <= 10; pin++) {

pinMode(pin, OUTPUT);

}

}

void setLights(bool ns_red, bool ns_yellow, bool ns_green, bool ew_red, bool ew_yellow, bool ew_green) {

digitalWrite(RED_NS, ns_red);

digitalWrite(YELLOW_NS, ns_yellow);

digitalWrite(GREEN_NS, ns_green);

digitalWrite(RED_EW, ew_red);

digitalWrite(YELLOW_EW, ew_yellow);

digitalWrite(GREEN_EW, ew_green);

}

void setup() {

initializeLights();

}

void loop() {

// North/South Red, East/West Green

setLights(true, false, false, false, false, true);

delay(GREEN_TIME);

// North/South Red, East/West Yellow

setLights(true, false, false, false, true, false);

delay(YELLOW_TIME);

// Both Red before switching

setLights(true, false, false, true, false, false);

delay(RED_TIME);

// East/West Red, North/South Green

setLights(false, false, true, true, false, false);

delay(GREEN_TIME);

// East/West Red, North/South Yellow

setLights(false, true, false, true, false, false);

delay(YELLOW_TIME);

// Both Red before switching back

setLights(true, false, false, true, false, false);

delay(RED_TIME);

}

D.  Demonstration Video

Record and upload a video demonstrating the operation of the circuit. (DONE)

Save the document as a PDF file and submit the PDF document to Blackboard.