Select one of the following projects and document your work with the following sections.  Provide a brief description of the system and how you are designing it to operate.

A.  Hardware Diagram:

Provide a hardware diagram of the components.

B.  Program Flowchart:

Draw a flowchart of the program.

C.  Arduino Source Code

Insert the Arduino Source Code into the document.

// ECE 2305 - Hardware Project 05

#include <Servo.h>

Servo voltmeterServo;

const int potPin = A0; // Potentiometer connected to A0

// Function to smooth noisy analog readings

int readStableAnalog(int pin) {

long sum = 0;

for (int i = 0; i < 10; i++) {

sum += analogRead(pin);

delay(1);

}

return sum / 10;

}

void setup() {

voltmeterServo.attach(9); // Servo connected to D9

}

void loop() {

int sensorValue = readStableAnalog(potPin);         // Read stable analog value

int angle = map(sensorValue, 0, 1023, 0, 180);       // Map 0–1023 to 0–180

static int lastAngle = -1;

if (abs(angle - lastAngle) > 5) {                   // Only update if angle changes > 2°

voltmeterServo.write(angle);                      // Move servo

lastAngle = angle;

}

delay(20);  // Smooth motion, reduce jitter

}

D.  Demonstration Video

Record and upload a video demonstrating the operation of the circuit. DONE

Save the document as a PDF file and submit the PDF document to Blackboard.

1.  Servo Voltmeter

Program an Arduino board to accept an analog input voltage and display the magnitude of the voltage using a Hobby Servo to display the voltage.  Use the servo horn to direct an arrow to the appropriate voltage.  Provide as much precision in the display as possible.