A.  Program Description:

The purpose of this program is to analyze the behavior of a simple RC circuit by computing the current and voltage across a capacitor over time. Given user input for time in microseconds, the program converts the input into seconds and calculates the corresponding values of current and voltage using exponential functions.

Inputs

- Time (t) in microseconds (entered by the user)

Outputs

- Current (i) in milliamps (mA)

- Voltage (v) in volts (V)

Program Structure

- The program uses sequential execution to define constants and read user input.

- It performs mathematical computations using predefined formulas.

- The final results are displayed using formatted output.

B.  Program Flowchart:

C.  Source Code

#include <iostream>

#include <cmath>

using namespace std;

int main()

{

// Constants

const double V_DC = 10.0;

const double R = 10000.0;

const double C = 100.0e-12;

double t_micro;  // Time in microseconds

// Prompt user for input

cout << "Enter time (t) in microseconds (us): ";

cin >> t_micro;

// Validate input (must be a number and positive)

if (cin.fail() || t_micro < 0) {

cout << "Invalid input. Please enter a positive numeric value." << endl;

return 1;

}

// Convert microseconds to seconds

double t = t_micro * 1e-6;

// Compute current (i) in mA

double i_t = (V_DC / R) * exp(-t / (R * C)) * 1000;

// Compute voltage (v) in V

double v_t = V_DC * (1 - exp(-t / (R * C)));

// Display results

cout << "Current, i(t): " << i_t << " mA" << endl;

cout << "Voltage, v(t): " << v_t << " V" << endl;

return 0;

}

D.  Program Test Results:

t = 0

t = 0.1

t = 0.2

t = 0.5

t = 1.0

t = 2.0

t = 5.0

E.  Tabulated Data.

| Time (t) (μS) | Current, i(t) (mA) | Voltage, v(t) (V) |
| --- | --- | --- |
| 0 | 1 mA | 0 V |
| 0.1 | 0.904837 mA | 0.951626 V |
| 0.2 | 0.818731 mA | 1.81269 V |
| 0.5 | 0.606531 mA | 3.93469 V |
| 1.0 | 0.367879 mA | 6.32121 V |
| 2.0 | 0.135335 mA | 8.64665 V |
| 5.0 | 0.00673795 mA | 9.93262 V |